State of Washington

Current Contract Information

Change No. 9

Effective Date: June 9, 2009

| Contract number: | 03404 |  | Commodity code: |  |  | 2325 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contract title: | ## TRUCK, INCIDENT RESPONSE VEHICLE |  |  |  |  |  |  |
| Purpose: | PRICE ADJUSTMENT |  |  |  |  |  |  |
| Current period: | May 17, 2009 | through: |  |  | May 16, 2010 |  |  |
| Contract type: | This contract is designated as convenience use. |  |  |  |  |  |  |
| # Scope of contract | This contract is awarded to one contractor(s). |  |  |  |  |  |  |
| Primary user: | Washington State Department of Transportation |  |  |  |  |  |  |
| Additional use by: | All State Agencies, Political Subdivisions of Washington and Oregon State, Qualified Non-profit Corporations, Materials Management Center, Participating Institutions of Higher Education (College and Universities, Community and Technical Colleges). |  |  |  |  |  |  |
| Contractor: | Titus-Will Ford |  |  | Contact: |  |  | Tunya Johnson |
| Address: | 3606 S. Sprague Ave Tacoma, WA 98409 |  |  | Phone: FAX: |  |  | (800) 637-6876 (253) 473-9250 |
| Email: | fleetexec@tituswill.com |  |  | Supplier No.: |  |  | 2430 |
|  |  |  |  | Fed. I.D. No.: |  |  | 91-0441515 |

This page contains key contract features.  Find detailed information on succeeding pages.  For more information on this contract, or if you have any questions, please contact your local agency Purchasing Office, or you may contact our office at the numbers listed below.

| Contract Specialist: | Corinna Cooper | Customer Service |  |
| --- | --- | --- | --- |
| Phone Number: | (360) 902-7422 | Phone Number: | (360) 902-7400 |
| Fax Number: | (360) 586-2426 | Fax Number: | (360) 586-4944 |
| Email: | clcoope@ga.wa.gov | Email: | csmail@ga.wa.gov |

Visit our Internet site: http://www.ga.wa.gov/purchase

| Products/Services available: | TRUCK, INCIDENT RESPONSE VEHICLE |  |  |  |
| --- | --- | --- | --- | --- |
| Ordering information: | See page 2 Note III |  |  |  |
| Ordering procedures: | See page 2 Note II |  |  |  |
| Special notes: | See Below |  |  |  |
| Payment address: | 3606 S. Sprague Ave Tacoma, WA 98409 |  |  |  |
| Order placement address: | 3606 S. Sprague Ave Tacoma, WA 98409 |  |  |  |
| Minimum orders: | None |  |  |  |
| Delivery time: | 220 days After Receipt of Order (ARO), See note below for explanation |  |  |  |
| Payment terms: | Net 30 days |  |  |  |
| Shipping destination: | Free On Board (FOB) Destination |  |  |  |
| Freight: | Prepaid and included in unit pricing |  |  |  |
| Contract pricing: | See pages Attachment “A” |  |  |  |
| Term worth: | $450,000.00 |  |  |  |
| Current participation: | $0.00 MBE | $0.00 WBE | $450,000.00 OTHER | $0.00 EXEMPT |
|  | MBE 0% | WBE 0% | OTHER 0% | Exempt 0% |

- Notes:

- Best Buy: The following provision applies to mandatory use contracts only.  This contract is subject to RCW 43.19.190(2) & RCW 43.19.1905(7): which authorizes state agencies to purchase materials, supplies, services, and equipment of equal quantity and quality to those on state contract from non-contract suppliers.  Provided that an agency subsequently notifies the Office of State Procurement (OSP) State Procurement Officer (SPO) that the pricing is less costly for such goods or services than the price from the state contractor.

- If the non-contract supplier's pricing is less, the state contractor shall be given the opportunity by the state agency to at least meet the non-contractor's price.  If the state contractor cannot meet the price, then the state agency may purchase the item(s) from the non-contract supplier, document the transactions on the appropriate form developed by OSP and forwarded to the SPO administering the state contract. (Reference General Authorities document)

- If a lower price can be identified on a repeated basis, the state reserves the right to renegotiate the pricing structure of this agreement.  In the event such negotiations fail, the state reserves the right to delete such item(s) from the contract.

- State Agencies: Submit Order directly to Contractor for processing.  Political Subdivisions: Submit orders directly to Contractor referencing State of Washington contract number.  If you are unsure of your status in the State Purchasing Cooperative call (360) 902-7415.

- Only authorized purchasers included in the State of Washington Purchasing Cooperative (WSPC) listings published and updated periodically by OSP may purchase from this contract.  It is the contractor’s responsibility to verify membership of these organizations prior to processing orders received under this contract.  A list of Washington members is available on the Internet https://fortress.wa.gov/ga/apps/coop/Default.aspx, Contractors shall not process state contract orders from unauthorized users.

- Percent increase to bid pricing for delivery to State of Oregon DASCPP/ORCPP  Members:   0 %

Special Conditions:

- Current Contract Information (CCI)

This Current Contract Information (Change Notice No. 9) has been issued for Titus-Will Ford to receive a price adjustment in accordance with the terms and conditions of the Contract.  Additionally, this contract works in conjunction with the Standard Terms and Conditions contained in the original IFB 03404 bid documents to include Special Provisions listed below per Section III, Specifications, Attachment “B”, and Change Notices 1 through 8.  All other Terms and Conditions remain unchanged.

- Certificate of Insurance

Contractor to furnish to the State Procurement Officer, a copy of a valid Certificate of Insurance listing the State as beneficiary (See Original Bid Document, Section II, Paragraph 1) within Fifteen (15) days after Contractor receipt of award notice.

- Warranty

Bidder will submit a copy of warranty with items delivered under this contract. See Section III, Specifications for warranty information. All materials or equipment provided shall be new, unused, of the latest model or design and of recent manufacture. See WARRANTY.

In the event of conflict between contract terms and conditions and warranty submitted, to afford the state maximum benefits, the contract terms and conditions shall prevail.

- Sales and Subcontractor Report

A quarterly Sales and Subcontractor Report (attached) shall be submitted in the format provided by the Office of State Procurement.  You can get the report electronically at https://fortress.wa.gov/ga/apps/CSR/login.aspx.  Total purchases for each State Agency, University, Community and Technical Colleges must be shown separately.  Total purchases for all political subdivisions and non-profit organizations may be summarized as one customer.  Additionally, all purchases by the State of Oregon or other purchasers must be reported as an aggregate total.

The report shall include sales information (Section A) and amounts paid to each subcontractor during the reporting period (Section B)

Reports should be rounded to nearest dollar.  Contractors will be provided with all necessary sample forms, instructions, and lists.  Reports are due thirty (30) days after the end of the calendar quarter, i.e., April 30th, July 31st, October 31st and January 31st.

## ATTACHMENT “A”

## PRICE SHEETS

| Req. Item | Comm. Code | Description | Unit | Unit Price |
| --- | --- | --- | --- | --- |
|  | - 2325 | - TRUCK INCIDENT RESPONSE W/UTILITY BODY. REFER TO SPECIFICATIONS SECTION 3.3, ATTACHMENT “B” BELOW. - Mfg.: FORD MOTOR COMPANY - Brand/Model: F450XL Note: See attached Warranty Attachment “B”. | - EA | - $102,329.03 |

OPTIONS 								PRICING:

- The right side shall a KWIK-RAZE 800 telescoping,

and removable light with tripod with a dedicated on/off switch.		ADD $1,662.00

- An Intelicom 1030 three-line VMS board to include

Installation and controls.                                                                          ADD $16,621.00

(Includes body reinforcement)

- Bidders shall provide a deduct amount for the specified 		DEDUCT $2,266.00

Arrow board.

- Cost to revise lighting per vehicle:					ADD $3,115.00

Options added to contract effective 6/20/2008:

1. Add on amount for quick disconnect feature of body from Chassis for all electrical components. $      614.00

2.  Add on amount for a 6 liter or greater Diesel engine.  Included in contract

3.  Add on amount to convert to all LED class 1 lighting. $ 3,580.00

4.  Add on amount for dual alternators for diesel engine.    $ 315.00

5.  Add on amount for a spring loaded cover to go over the flair dispenser.  $ 30.00

6.  Add on amount for a Traffic Manager SYS3823060L in lieu of the National Signal arrow board. (See #3)

7.   Add on amount to relocate the second battery to the compartment where the Generator is and located above the generator. $ 268.00

8.  Add on amount for Male hydraulic fittings on the suction side of the fuel transfer system $42.00 each

9.  Add on amount for Female hydraulic fittings on the discharge side of the fuel transfer system. $42.00

10. Add a canister filter to the on both suction and discharge lines and marked for flow direction. $241.00

11. Add on amount for a discharge nozzle with the ability to be used on the 3 foot suction hose annotated in item X.11. $48.00

12.  Add on amount for the relocation of the Ametek grounding and bonding reel to the left rear compartment. $No charge.

13.  Add on amount for a fuel level gauge mounted in the cab which will provide fuel levels in the fuel transfer storage tank. $187.00

14.  Add on amount to relocate the current fuel level sight gauge to the left rear storage corner in lieu of the center of the tank. $No charge

15.  Add on amount to increase the size of the expanded metal protection around the sight gauge.  Require the next upgraded gauge of expanded metal. $30.00

16. Add on amount for a standard sliding roof style service body with one vertical, one horizontal, and one vertical compartment per side. Not available on this contract.

17.  Add on amount for the Traffic Manager SYS3823060L to be mounted on the cab and in front of the standard service body. Not available on this contract.

Washington State

Department of Transportation

### Vehicle & Equipment Bidding Requirements

- SPECIFICATIONS

Acceptance of Terms:

Acceptance of a state Purchase Order (PO) for any units affiliated with this purchase constitutes acceptance of, and agreement with, all of the general and specific requirements and stipulations listed in this boiler plate, and in the attached equipment specification(s); including all penalties mentioned.

General:

Units bid for this purchase must be new (unused), current production models that require no manufacturer or dealership modifications.   Units may be sold, prepared, and delivered to WSDOT, or a designated agent, only by a dealer who is factory franchised for the specific makes and models of equipment offered.

All accessories and features listed herein shall be those supplied by the original equipment manufacturer (OEM).  Any accessories, features, or operational performance required by FMVSS, Washington State Motor Vehicle Laws, OSHA or WISHA laws or mandates, that apply to the equipment being bid, shall be provided by the manufacturer.   All units associated with this purchase shall be of the same design and quality as those sold through normal retail channels; and they shall possess the latest technology, accessories, and features offered on standard retail unit; whether or not they are called for in the following specifications.

Specification Clarification and Changes:

Clarification for any item in these specifications may be obtained from the Office of State Procurement, (OSP) at (360) 902-7422.

The equipment specification(s) for this purchase are official state documents that carry far reaching ethical and legal implications.  Therefore, after a purchase order is awarded to a successful bidder, there shall be no deviations from any requirements stated in the published equipment specification(s) during the manufacturing or assembly process of the units offered, without prior approval from the WSDOT Equipment Manager, and an official purchase order change issued by the OSP.  Failure to comply with this requirement constitutes breach of contract; and may be grounds for order cancellation, without re-stocking fees or damages to WSDOT; or suspension from the state bidders list.

Equipment Demonstrations:

Prior to award, bidders may be required to present an offered unit for demonstration of its performance and capability.   Such demonstrations must be conducted within seven calendar days after notification.

Equipment Specification Compliance Inspections; Delivery;  and Acceptance:

To minimize time involvement and transportation costs to resolve equipment non-compliance issues, WSDOT will conduct specification compliance inspections at the supplier’s location prior to equipment delivery.   Suppliers must contact the WSDOT Equipment Administration Office, at (360) 705-7869, to arrange for compliance inspections.  To allow for a reasonable reaction time, this notification must be at least 5 working days prior to a desired inspection date.

Equipment suppliers are responsible for ensuring that all units comply with all of the requirements listed in the boilerplate and specification(s) of this purchase, prior to a compliance inspection.  This includes presenting the following documentation for each unit during the inspection:

- The manufacturer’s line production sheet reflecting the equipment serial numbers, and listing all of the unit’s components

- Completed Invoice

- The Manufacturer’s Statement of Origin  (MSO)

- Axle weight slips  (for all units with axles)

- The title application reflecting both the legal and registered owner as:

Washington State Department of Transportation

Olympia, Washington   98504-7357

When equipment fails to meet a compliance inspection, the supplier hereby agrees to reimburse WSDOT for any travel, per diem, and labor costs associated with all necessary re-inspection(s).  Such reimbursement shall be deducted from the invoiced amount at final payment.

Equipment shall not be delivered to a WSDOT location until this inspection has successfully occurred, and a signed copy of the WSDOT specification compliance worksheet has been issued to the supplier.   Failure to comply with this specific requirement constitutes breach of contract; and may be grounds for order cancellation, without re-stocking fees or damages to WSDOT; or suspension from the state bidders list.

After a unit successfully passes a compliance inspection, it may be delivered to WSDOT facilities between the hours of 7:00 am and 3:00 pm – Monday through Friday.    Deliveries shall not occur during other hours; or on weekends; or on recognized state and federal holidays.   Suppliers must notify the phone number annotated in the ship to block of the purchase order at least 24 clock hours prior to equipment delivery, to ensure that a WSDOT employee is available to sign and date the delivery acceptance of the unit(s).   Equipment left at a WSDOT facility without being signed for by an appropriate employee, will not be considered accepted by WSDOT; and WSDOT will not accept responsibility for the safeguard of such equipment.  Failure to comply with this requirement constitutes breach of contract; and may be grounds for suspension from the state bidders list.

Late Delivery Penalty:

Receiving new units within the fiscal year for which they are funded is critical to government agencies.   Therefore, WSDOT reserves the right to accept or reject bids based solely upon the bid delivery date.

A late delivery penalty of two tenths of one percent (0.002) shall be levied against the bid price of each individual unit, for each normal workday beyond the delivery date reflected on the awarded Purchase Order.   Late penalties shall stop upon delivery acceptance annotated on the previous page of this document.  WSDOT shall deduct such late delivery penalties from the invoiced amount, when making payment.   Officially recognized state and federal holidays shall not be considered normal workdays.    Suppliers may not be held responsible for Force Majeure delays; providing the OSP procurement administrator is notified in writing within 10 calendar days after the Force Majeure event.  The OSP Procurement Administrator is responsible for consulting the WSDOT Fleet & Equipment Manager on such events; and the WSDOT Fleet & Equipment Manager has the sole prerogative of determining if events fall into the category of Force Majeure.

For sensible reasons, suppliers may request relief of late penalty fees by contacting the WSDOT Fleet & Equipment Manger in writing.   Under no circumstances may any other person excuse late delivery penalty.

Warranty Services and Performance:

Equipment suppliers must provide technical support and reasonable equipment modifications for a period of 90 calendar days after the unit is reported as in service, to ensure that the purchased equipment is capable of performing the specified operational functions.

Bidders must include the factory warranty, which shall cover 100% parts and labor for the entire unit offered.   This warranty must be honored at any factory-franchised dealership.

Warranty coverage will not commence until the date the completed unit is put into service as reported by WSDOT; or 30 days after final payment for the unit(s); whichever occurs first.

The equipment supplier must be capable of providing repair parts and supplies support for a period of at least 10 years after the purchase of the unit(s) offered.

*Bidder can only guarantee if vehicle is brought to Titus Will Ford for repair.

The equipment supplier must initiate physical repairs on equipment failures within 72 consecutive hours after notification, during the purchased equipment’s warranty period.

Suppliers shall incur all costs associated with warranty work for units obtained from this purchase, during the warranty period; this includes the transport of units that are disabled due to the failure of a warranted item.

*Per Ford Motor Company 3 years/36,000  mile warranty.

Suppliers may authorize WSDOT to accomplish warranty repairs upon request; however, the supplier also agrees to wholly reimburse WSDOT for parts, materials, labor, and travel costs incurred in the accomplishment of such warranty repairs.   The billing document for warranty work performed by WSDOT shall be based upon a WSDOT shop service work order for claims reimbursement.

WSDOT may levy charges equal to damages incurred as a result of non-compliance with any of the requirements in this Warranty Services and Performance section.   These damages may include such things as lost productivity, and penalties levied on WSDOT by a third party.

*All damages due to lost productivity and penalties by 3rd party must be directed to Ford Motor Company pertaining to chassis, and to Northend Truck Equipment pertaining to the body.

Training:

Equipment suppliers shall provide one session of operator training per unit purchased; and one session of mechanic training per unit purchased.  The session will include, but not limited to, the training items listed below.

- Operator training will be designed to familiarize personnel with operating characteristics and operator checks of the new equipment.    This will include teaching operators shifting, acceleration, and braking techniques to maximize operational effectiveness of the unit’s power train configuration.

- Mechanic training shall be designed to familiarize service and repair technicians with service checks and techniques, adjustments, and any unique requirements associated with the entire unit.

All training is to be scheduled, and coordinated by the WSDOT, ship to addressee.

Training sessions shall be conducted by qualified individuals only.  “Qualified” means that the trainer must have a high level of knowledge and experience relating to the type of equipment offered or purchased:

- Persons conducting the operator sessions must have a minimum of one (1) year of experience in actually operating the units offered.

- Persons conducting mechanic training must have at least one (1) year of experience in the performance of preventive maintenance and repair on the units offered.

Training sessions will be evaluated by the agency Equipment Training Manager, who shall determine weather or not the training was adequate.   If the training is deemed inadequate, the supplier agrees to conduct additional training sessions, at no cost, to satisfy the requirement.

| 0301 | # Truck; Incident Response; Utility Body |
| --- | --- |

| Specification Requirements |  | Check If Meet or Exceed |  | Describe Offered Alternatives Revised 5/14/07 |
| --- | --- | --- | --- | --- |
| - General: |  |  |  |  |
| - This specification intends to describe the Department of Transportation’s requirements for a highway Incident Response Vehicle |  | X |  |  |
| - Cab and Chassis: |  |  |  |  |
| - The chassis shall be a 4X4 heavy duty extended cab, 15,000 GVWR with dual rear wheels and tires. |  | X |  | Revised 5/14/07: 16,000 GVWR |
| - A 6-liter or greater gasoline engine, with electronic overdrive automatic transmission and a 4:10 to 1 differential. |  |  |  | Revised 5/14/07: 6.4 Liter OHV Powerstroke Diesel, V8 Engine. 4.88 required axle ratio. |
| - Manufacturer installed engine and transmission oil coolers. |  | X |  |  |
| - Four wheel power ABS disc brakes. |  | X |  |  |
| - HD shocks, Front & Rear Stabilizers, and a class III Tow Package. |  | X |  | Revised 5/14/07: Trailer Tow Pkg includes 7 wire harness w/relays, blunt cut & labeled. After market trailer brake wiring kit. TT guide & trailer brake controller not included. |
| - Electric Intermittent windshield wipers with washer capability. |  | X |  |  |
| - Dual cloth covered bucket seats with fold down armrest. |  |  |  | Revised 5/14/07: Fold down armrests not available. HD cloth bucket Vinyl rear bench seat. |
| - Black rubber floor mats. |  | X |  |  |
| - A standard AM/FM radio w/clock. |  | X |  |  |
| - Power Steering |  | X |  |  |
| - Power Door Locks |  | X |  |  |
| - Power Windows |  | X |  | Revised 5/14/07: Factory installed 200 amp, alternator |
| - A 140+ Amp. Alternator. |  | X |  | Revised 5/2008: Factory installed 200 Amp Alternator. |
| - Dual 750CCA batteries mounted in the engine compartment. |  | X |  | Revised 5/14/07: Standard with diesel engine. |
| - The manufacturers standard gauge package. |  | X |  |  |
| - The maximum fuel capacity for the truck model. |  | X |  | 40 gallon. |
| - Factory-installed air conditioning. |  | X |  |  |
| - All season mud and snow radial tires matching truck GVWR with the a speed rating of 70 MPH or greater. |  |  |  | Revised 5/14/07: 225/70R x 19.56 BSW All season tires w/matching spare. |
| - A full size matching spare tire and wheel. |  | X |  |  |
| - Left and right extendable and breakaway external rear view mirrors with electrical defrost and remote adjust |  | X |  |  |
| - A formed black bug deflector mounted on the trucks hood. |  | X |  | Dealer installed smoked bug guard. |
| - Shall be equipped with running boards extending from the rear of the front fender well to the service body. |  | X |  |  |
| - All exterior windows shall be tinted safety glass. |  | X |  | Solar Tint. |
| - Cab Control Console: |  |  |  |  |
| - Shall be equipped with a control console for all radios annotated in this specification. |  | X |  |  |
| - The forward portion of the console (Radio mounting locations) will be angled upward to enhance radio visibility for the operator. |  | X |  |  |
| - The console shall have a removable top to accommodate maintenance of the internal components. |  | X |  |  |
| - Shall have a writing surface and storage compartment: measuring 9”x11” to include a hinged door for the storage compartment. The writing surface shall be located as far forward as possible. |  | X |  |  |
| - Shall have four (4) 12-volt auxiliary power receptacles mounted on the forward end of the console and accessible to the operator. |  | X |  |  |
| - Shall have a (Little-lite L5/18) map light mounted on the front edge of the console and to the left of center. |  | X |  |  |
| - For those units with an installed arrow board, the controller shall be mounted on the right front of the console and angled toward the operator. |  | X |  |  |
| - Shall have a 12 Volt road flare dispenser mounted between the driver’s seat and the control console. The top of the dispenser will be level with the drivers seat cushion. |  | X |  |  |
| - A twelve (12)-road flare-holding bracket shall be mounted in such a manor, that it will allow the operator to obtain flares while driving. |  | X |  |  |
| - The console shall have permanently labeled and lighted rocker panel switches. |  | X |  |  |
| - The rocker panel switches shall be International part number 1677051C1. Below are the console-mounted switches and how they will be labeled. The location of each switch shall start at the upper left and proceed from left to right then down. |  | X |  |  |
| Top Row |  | X |  |  |
| - Front and rear amber flasher |  | X |  |  |
| - Side red flasher |  | X |  |  |
| - Red rear flasher |  | X |  |  |
| - Front grill alt. flasher |  | X |  |  |
| - Light Bar (Two Switches) |  | X |  |  |
| 1. Take Down |  | X |  |  |
| 2. Rotating |  | X |  |  |
| - Wig Wag Headlights |  | X |  |  |
| Second Row |  | X |  |  |
| a. Left Alley |  | X |  |  |
| b. Left Flood |  | X |  |  |
| c. Right Flood |  | X |  |  |
| d. Right Alley |  |  |  |  |
| - Arrow Board indicator Light (Red is board movement/Green is in the raised position. / no indicator when in stowed position. |  | X |  |  |
| - Rear Camera. |  | X |  |  |
| g. Rear Work Lights |  | X |  |  |
| - Radio Installation: |  |  |  |  |
| - The successful bidder shall provide and professionally install all wiring, for the following radios and their speakers. The radios described in “a” and “b” will be supplied by WSDOT. |  | X |  |  |
| - High Band Radio. |  | X |  | DOT Supplied. |
| - E .F. Johnson Radio. |  | X |  | DOT Supplied. |
| - CB Radio (Uniden PRO0510XL) |  | X |  |  |
| - At the main fuse panel, there shall be a labeled, dedicated, and fused, 15-amp constant duty circuit for the EFJ Radio. This circuit shall run directly from a battery power source. At the same location, there shall be a labeled 3 amp fused circuit. This circuit shall be switched at the ignition switch, and shall operate in the run or accessory positions. |  | X |  |  |
| - Connected to the 15 amp EFJ circuit shall be a power wire with the same connectors, gauge, and length of the EFJ Part# 023-9750. These wires shall be labeled EFJ Radio, and terminate at the radio. |  | X |  |  |
| - Connected to the 3-amp EFJ circuit shall be a sense wire, with the same connectors, gauge, and length of the EFJ Part # 023-9750-011. These wires shall be labeled EFJ Radio and terminate at the radio |  | X |  |  |
| - The CB radio and wiring shall be supplied by the successful bidder and shall be a Uniden PRO0510XL. These wires shall be fused and labeled CB Radio and terminate at the radio. |  | X |  |  |
| - Vendor will supply a fused and labeled wiring harness for the high band radio with the wire terminating at the radio mounting location. |  | X |  |  |
| Note: WSDOT may temporarily loan its radios to the successful bidder to aid in determining measurements for proper fit. |  |  |  |  |
| - Each radio shall have a dedicated functional speaker. The speakers shall be professionally mounted in the cab directly below the rear window. |  | X |  |  |
| - Three Larson NMO-K-DS-FME radio antenna bases with protective caps shall be provided and installed on the upper portion of the service body. |  | X |  |  |
| - The antenna cables shall extend to the control console and have 12 inches of excess cable. |  | X |  |  |
| - Shall be equipped with three microphone clips that will be installed on the left side of the control console. |  | X |  |  |
| - Camera System: |  |  |  |  |
| - The unit shall have a rear view ASA Voyager Model VCC130 color camera. |  | X |  |  |
| - The camera shall be mounted at the rear of the service body, and centered above the rear doors. |  | X |  |  |
| - Shall have a Voyager model FDK681CSPL color monitor installed in the cab for viewing by the operator while driving. Supplier shall not use the computer mount. |  | X |  |  |
| - The monitor shall be mounted directly below the arrow board controller and angled towards the operator. |  | X |  |  |
| - The camera system will be activated by a switch mounted on the console with or without the ignition on. |  | X |  |  |
| - Shall have work lights mounted on the rear of the service body, at upper left and right corners to provide lighting for the camera system. Lights shall be operated from the cab control console via switch with the ignition off and when the unit is in reverse. |  | X |  |  |
|  |  | X |  |  |
| - Front Push Bumper: |  |  |  |  |
| - The truck shall have a front push bumper installed that is capable of pushing vehicles or objects weighing up to 15,000 pounds. |  | X |  |  |
| - The bumper must wrap around the front of the truck. |  | X |  |  |
| - The bumper surface shall be, completely finished with a ½ -inch seamless Black Teflon (Not Painted) material. Additionally; all mounting bolts will be recessed into the bumper then covered with a rosette. |  | X |  |  |
| - The bumper shall be 14 inches above the ground and shall not extend past the bottom of the grill with an overall height not to exceed 34 inches. |  | X |  |  |
| 5. The bumper will be vented to ensure the chassis engine receives the proper amount of airflow during operation. |  | X |  |  |
| 6. The bumper shall include a 1-¾-inch OD schedule 80 tube running full width, and elevated 12 inches from the top of the bumper. The tube shall follow the contour of the bumper and shall include five vertical supports constructed from the same 1-¾ inch material. |  | X |  |  |
| 7. Shall have one (Safe-t Connect-t Starting Kit) Part No. 394017003 jumper terminals installed on the upper right corner of the front bumper. These terminals shall not interfere with pushing functions. |  | X |  |  |
| 8. All front bumper steel shall be painted black. |  | X |  |  |
| - Emergency Lighting: |  |  |  |  |
| - Shall be equipped with a Street Hawk LED light bar, without PA system. |  | X |  |  |
| - The light bar shall be mounted on the front of the service body and four inches above the chassis cab. |  | X |  |  |
| - The forward top portion of the service body roof shall be equipped with a cab controlled 360-degree rotate able and amiable spot light. |  | X |  |  |
| - Shall have a PA system, and siren located under the hood and as close to the front grill as possible |  | X |  |  |
| - Shall have two, fixed, 6-inch, red LED flashing lights, mounted on each upper side corners of the service body. |  | X |  |  |
| - Shall have two -fixed 6-inch amber LED flashing lights mounted on the upper front and rear of the service body. |  | X |  |  |
| - Shall have two -fixed 6-inch red LED flashing lights mounted on the upper rear of the service body and above the two amber lights. |  | X |  |  |
| - The left and right side of the body shall be equipped with a Fire Research Model LT540, 500 watt 120 volt extendible floodlight. |  | X |  |  |
| - The floodlights shall be mounted on each front corner of the service body and shall receive their power from the Honda generator. |  | X |  |  |
| - The unit shall be equipped with a LED arrow board. The arrow board will raise and lower using an electric worm gear Dayton actuator with cab controls. |  | X |  |  |
| - The arrow board shall be mounted on top of, and at the rear of the service body. |  | X |  |  |
| - When the board is in the lowered position, it shall not extend beyond the service body. |  | X |  |  |
| - The service body shall be reinforced at the arrow board mounting points. |  | X |  |  |
| - The arrow board mounting structure will ensure vibration is reduced, and will have the strength to allow the sign to be in the raised position while driving at 35 MPH |  | X |  |  |
| - When the arrow board is in the lowered position the light pockets shall face downward and shall have a lowered height of 6 to 8 inches. |  | X |  |  |
| - Shall be equipped with two (2) four inch red alternating, LED flashing lights mounted above the front bumper. The lights shall be located on the center two bumper support struts and evenly spaced. |  | X |  |  |
| - Shall have a wig wag headlight lighting system |  | X |  |  |
| - Service Body: |  |  |  |  |
| - The service body shall be a Reading PA60-1086DW or equal. |  | X |  | Monroe Custom Body Equal. Model # S SB 108 M 6017-T |
| - All body compartment and seams shall be fully seam welded. |  | X |  |  |
| - The body shall be equipped with an exhaust fan with the ability to constantly run when the ignition is on. The fan will be covered with a Max Air Vent cover. |  | X |  |  |
| - The interior of the body shall be vented or heated to prevent moisture from developing. |  | X |  |  |
| - The entire perimeter of the sub-floor shall be completely welded creating a watertight environment. |  | X |  |  |
| - Compartments: |  |  |  |  |
| - All vertical compartment doors shall open out and to the front, and shall be equipped with a spring type-locking device. |  | X |  |  |
| - The horizontal doors shall be bottom hinged and equipped with chain stops. The chain stops will not interfere with the operation of the doors. |  | X |  |  |
| - All door chain stops shall have full-length rubber protective sleeves. |  | X |  |  |
| - All compartment and rear door handles shall be “D” ring twist. |  | X |  |  |
| - A drain hole shall be provided in all exterior compartment bottoms. The drain shall be equipped with a self-draining plug. |  | X |  |  |
| - All compartment floors shall be lined with a quality grade rubber mat. |  | X |  |  |
| - A Vista-Track lighting system shall be installed in all outside compartments and shall be activated by a Cole Hersee 91823 door jamb switch, |  | X |  |  |
| - All service body door locks shall have CH 501 lock cylinders |  | X |  |  |
| - All key s shall be oriented with the teeth facing up. |  | X |  |  |
| - All compartment shelving shall have a 200-pound capacity. |  | X |  |  |
| - Curbside Compartments: |  |  |  |  |
| - A Honda EU 2000 generator shall be supplied and installed in the front curbside vertical compartment on a, slide out tray with easy off mounting devices. The tray will lock in the open and closed positions using a push button locking device. |  | X |  |  |
| - The generator compartment shall have adjustable shelving above the generator. |  | X |  |  |
| - The generator compartment shall be vented to accommodate the engine exhaust. The exhaust fumes shall not enter the service body or the chassis cab. |  | X |  |  |
| - The Honda generator shall have the ability to run at maximum capacity with the compartment door closed without damage to the generator. |  | X |  |  |
| - The curbside horizontal compartment shall be equipped with a full-length adjustable height shelf. |  | X |  |  |
| - The curbside rear compartment shall be equipped with a 12 volt Fill Rite 18 GPM fuel transfer pump system providing loading and off-loading of the on-board storage tank and transferring fuel between vehicles. |  | X |  |  |
| - The pump shall be mounted in the upper one third of the compartment leaving the lower two thirds for hose storage. |  | X |  |  |
| - The fuel transfer pump system shall have the ball valve(s) necessary to provide the following operations. |  | X |  |  |
| - The ball valve handles will be color coded as follows Suction (Blue) Discharge (White) |  | X |  |  |
| - Bumper suction to storage tank. |  | X |  |  |
| - Bumper suction to bumper discharge. |  | X |  |  |
| - Storage tank to bumper discharge. |  | X |  |  |
| - The fittings at the rear bumper shall be permanently labeled with the color code for each function |  | X |  |  |
| - The unit shall be equipped with one (1) loose 12 foot suction hose and one (1) loose 12 foot return hose with quick disconnect fittings on both ends, and one quick disconnect discharge nozzle. Each hose will be color coded for each function (Suction, Discharge) |  | X |  |  |
| - One rubber pick-up hose with a clear canister type filter and connections for use with (10) above to load or transfer fuel. |  | X |  |  |
| - Shall have suction and return lines plumbed from the pump to the 100 gal. fuel storage tank. |  | X |  |  |
| - All hydraulic hoses and fittings will be one inch. |  | X |  |  |
| - These lines shall not run on the interior of the service body. |  | X |  |  |
| - The lines shall be protected with split loom and protected from chafing when running through or around any metal objects. |  | X |  |  |
| - The unit shall have grounding and bonding system installed. |  | X |  |  |
| - Shall have an Ametek model 700-50R grounding reel mounted on the left side of the rear bumper. |  | X |  |  |
| - The Ametek reel shall be grounded to the chassis frame, using a braded steel-grounding strap. |  | X |  |  |
| - Shall have a Lab Safety SPQD 00173544 grounding coil with clamps for truck to truck bonding. |  | X |  |  |
| - Street side Compartment: |  |  |  |  |
| - The streetside front and rear vertical compartments shall be outfitted with adjustable shelving running full height. |  | X |  |  |
| - The streetside horizontal compartment shall be equipped with a full-length adjustable shelf. |  | X |  |  |
| - Rear Exterior Access Doors: |  |  |  |  |
| - The rear doors shall be the manufacturers standard and be weather tight . |  | X |  |  |
| - The rear doors shall use gas struts to keep the doors in the open position. |  | X |  |  |
| - The rear entrance doors shall have an interior and exterior handle. |  | X |  |  |
| - Interior Service Body Configuration: |  |  |  |  |
| - The interior roof shall be equipped with three, (3) equally spaced, incandescent 6 inch round lights. |  | X |  |  |
| - The lights shall have dual switching capability. |  | X |  |  |
| - Door jam switch |  | X |  |  |
| - Shall have a right rear, interior wall-mounted three-way switch, which will work in conjunction with the door jam switch. |  | X |  |  |
| - There shall be a 100 gal. Aluminum fuel storage tank located in the front of the service body and mounted to the floor. |  | X |  |  |
| - The fuel storage tank shall be plumbed to the fuel transfer pump system. |  | X |  |  |
| - The fuel storage tank shall run full width of the service body and shall be vented to the outside. |  | X |  |  |
| - The fuel storage tank shall be equipped with a sight gauge mounted on the center of the tank, and shall be visible from the rear of the truck. Additionally; the sight gauge shall be protected with an expanded metal cage. |  | X |  |  |
| - Mounted two inches above the fuel tank, shall be a full width, 500 pound, 24 inch deep shelf. The shelf shall have a two-inch lip, on the outer edge to prevent items from falling onto the service body floor. |  | X |  |  |
| - Located on the curbside interior floor shall be twelve (12) 5-inch square aluminum (Honey Comb) sign pockets, mounted vertically in rows of two. (6 in each row). |  | X |  |  |
| - The ends of the pockets facing the rear door shall be open and the ends facing the cab shall be sealed. |  | X |  |  |
| - Shall be equipped with one row of eight (8) traffic sign stands mounted on the street side interior of the service body. |  | X |  |  |
| - The stands shall be constructed of 1-inch OD square steel tube, 4 inches in height, spaced eight (8) inches on center and welded to a steel plate which will be bolted to the service body floor. |  | X |  |  |
| - Rear Bumper: |  |  |  |  |
| - Step/bumper and frame shall be full width and constructed of mild steel . |  | X |  |  |
| - Shall be equipped with non-skid paint along the entire surface of the bumper. |  | X |  |  |
| - The step bumper shall be reinforced and outfitted with a class III towing hitch, to include a 2-inch trailer ball, a 3-inch pintle eye combination and a 7-pole trailer connector. |  | X |  |  |
| - The rear step/bumper shall not have a cut out for the trailer towing pintle. The receiver will be flush with the rear bumper. |  | X |  |  |
| - The bumper will be outfitted with a tuck away rear step, for access into the service body. |  | X |  |  |
| - Shall have one (Safe-t Connect-t Starting Kit) Part No. 394017003 jumper terminals installed on top of the right corner of the rear step bumper. These terminals shall not interfere with towing functions. |  | X |  |  |
| - Electrical: |  |  |  |  |
| - All added body and chassis electrical circuits shall serve equipment separate and distinct from the vehicle chassis circuits. Additionally all wiring shall be shielded to prevent electronic interference with the communication system. |  | X |  |  |
| - All non-OEM wiring shall be copper. |  | X |  |  |
| - The wiring shall be color, function, and number coded every 6 inches for permanent identification and corresponds with the vehicle schematics. |  | X |  |  |
| - Soldered and insulated connectors shall be used. |  | X |  |  |
| - All wiring shall be located in accessible, enclosed, and protected locations. |  | X |  |  |
| - All wiring shall be routed in split loom and secured with insulated metal cable straps (Del Clamps). Rubber grommets shall be used when passing through holes. |  | X |  |  |
| - Where wiring passes over the exhaust, a heat shield shall be installed. |  | X |  |  |
| - A 6-inch service loop of wire or harness shall be provided at all electrical components, terminals, and connection points. |  | X |  |  |
| - The circuit breakers, relays, and other electrical items shall be located in the enclosed power component electrical panel. |  | X |  |  |
| - The electrical panel shall be located behind the drivers seat. |  | X |  |  |
| - The entire front door of the cabinet shall be clear Plexiglas. |  | X |  |  |
| - The cabinet shall be mounted to the chassis rear interior wall utilizing an isolating type material. |  | X |  |  |
| - All circuits shall be labeled identifying the specific function. |  | X |  |  |
| - Two (2) complete sets of detailed wiring schematics shall be provided as well as one (1) individual drawing, breaking down portions of the schematics. |  | X |  |  |
| - A detailed wire harness list shall be provided, showing termination points along with isometric views of the cab and module illustrating harness routing. |  | X |  |  |
| - A materials list with code letters, which correspond with the schematics, and includes the location of every switch and component shall be provided. |  | X |  |  |
| - A module disconnect switch shall be wired to remove the electrical load simultaneously from all other electronic equipment. |  | X |  |  |
| - Shall have an electric auto throttle, activated by a voltage monitoring system. This system shall use the aux. battery for its power |  | X |  |  |
| - When the transmission is in park or neutral, the throttle will activate when voltage drops below 11.5 volts and deactivate at 13.5 volts. |  | X |  |  |
| - This system shall not rely on engine vacuum to operate. |  | X |  |  |
| - In addition to OEM grounds. The following ground circuits shall be added. |  | X |  |  |
| - 4-gauge ground cable from module power component panel to frame. |  | X |  |  |
| - 2/0 gauge ground cable from the engine block to frame. |  | X |  |  |
| - 2/0 gauge ground cable from batteries to frame |  | X |  |  |
| - The service body shall be grounded to the chassis frame |  | X |  |  |
| - Painting, and Decals: |  |  |  |  |
| - The exterior finish is expected to be superior craftsmanship. There shall be no welding scale, rough, sharp edges or corners. |  | X |  |  |
| - The chassis and underside of the service body shall be painted with the manufacture's standard black paint and undercoated. |  | X |  |  |
| - The cab exterior shall be painted with the manufacture's standard primer and white paint. |  | X |  |  |
| - The Service Body shall be prime coated with 3 mils of dry automotive quality primer, and 3 mils of white paint matching the cab's color. |  | X |  |  |
| - Each unit shall be outfitted with the WSDOT's incident response vehicle decals, prior to the compliance inspection. The supplier will notify Billboard Signs at 206-300-7300 for decal installation. Billboard Signs is responsible of the correct installation of WSDOT decal. Billboard Signs will bill WSDOT for decals and decal installation. |  | X |  |  |
| - The phrase INCIDENT RESPONSE shall be applied on the front deflector shield in 4-inch reflective white block lettering. These letters shall be installed so that the phrase can be correctly read when seeing it through a rear view mirror. |  | X |  |  |
| - The entire unit shall be painted white. |  | X |  |  |
| - The phrase INCIDENT RESPONSE shall be applied on the rear center of the service body in 4-inch reflective black block lettering. |  | X |  |  |
| - Publications: |  |  |  |  |
| - Each unit shall be delivered with an operator’s manual. |  | X |  | As applies to Body |
| - Bidders shall provide the total quantity of Service, and Parts Manuals as annotated below. 2 Parts Manual (Per Unit) 2 Service Manual (per Unit |  | X |  | As applies to Body |
| XVIII User Funded Additional Needs: |  |  |  |  |
| All items annotated below, will be individually priced, and separate from the base bid. |  | X |  |  |
| 1. The right side shall a KWIK-RAZE 800 telescoping, and removable light with tripod with a dedicated on/off switch |  | X |  | Add $1,662.00 |
| 2. An Intelicom 1030 three-line VMS board to include installation and controls. |  | X |  | Add $16,621.00 (Includes body reinforcement) |
| 3. Bidders shall provide a deduct amount for the specified arrow board. |  | X |  | Deduct -($2,266.00) |
| 4. The service body will be reinforced at the VMS board mounting locations, to allow the board to be in the raised position while traveling at 35 MPH |  | X |  |  |
| 5. LED Light Package: a. The light bar mounted to front of body. b. The arrow board mounted on top of body @ rear. c. Replace all other emergency flashing lights w/ comparable LED version. |  | X |  | $3,115.00 |

PERFORMANCE REPORT FOR

Purchasing & Contract Administration

To OSP Customers:

Please take a moment to let us know how our services have measured up to your expectations on this contract.  Please copy this form locally as needed and forward to the Office of State Procurement Purchasing Manager.  For any comments marked unacceptable, please explain in remarks block.

| Procurement services provided: | Excellent | Good | Acceptable | Unacceptable |
| --- | --- | --- | --- | --- |
| - Timeliness of contract actions |  |  |  |  |
| - Professionalism and courtesy of staff |  |  |  |  |
| - Services provided met customer needs |  |  |  |  |
| - Knowledge of procurement rules and regulations |  |  |  |  |
| - Responsiveness/problem resolution |  |  |  |  |
| - Timely and effective communications |  |  |  |  |

Comments:

| Agency: |  | Prepared by: |  |
| --- | --- | --- | --- |
| Contract No.: | 03404 | Title: |  |
| Contract Title: | TRUCK, INCIDENT RESPONSE VEHICLE | Date: |  |
|  |  | Phone: |  |

Send to:

Unit Manager

Office of State Procurement

PO Box 41017

Olympia, Washington 98504-1017

PERFORMANCE REPORT FOR

CONTRACTOR PRODUCT/SERVICE

Complete this form to report problems with suppliers or to report unsatisfactory product or services.  You are also encouraged to report superior performance.  Agency personnel should contact suppliers in an effort to resolve problems themselves prior to completion and submission of this report.

Contract number and title:

Supplier’s name: 						 Supplier’s representative:

| PRODUCT/SERVICE |  |  |  |
| --- | --- | --- | --- |
|  | Contract item quality higher than required |  | Damaged goods delivered |
|  | Contract item quality lower than required. |  | Item delivered does not meet P.O./contract specifications |
|  | Other: |  |  |
| SUPPLIER/CONTRACTOR PERFORMANCE |  |  |  |
|  | Late delivery |  | Slow response to problems and problem resolution |
|  | Incorrect invoice pricing. |  | Superior performance |
|  | Other: |  |  |
| CONTRACT PROVISIONS |  |  |  |
|  | Terms and conditions inadequate |  | Additional items or services are required. |
|  | Specifications need to be revised |  | Minimum order too high. |
|  | Other: |  |  |

Briefly describe situation:

| Agency Name: |  | Delivery Location: |  |
| --- | --- | --- | --- |
| Prepared By: | Phone Number: | Date: | Supervisor: |
| Address: | Email: |  |  |

Send To:				STATE PROCUREMENT OFFICER

OFFICE OF STATE PROCUREMENT

PO BOX 41017

OLYMPIA WA  98504-1017