# APPENDIX C

# TECHNICAL REQUIREMENTS – Revision 1

- SHIPPING TERMS

| Terms of Sale | Responsibility for Freight Cost and Transit Risk |
| --- | --- |
| F.O.B. Destination, freight prepaid and included | Seller - Pays freight charges Seller - Bears freight charges Seller - Owns goods in transit Seller - Files claims (if any) |

- VEHICLE AND BIDDING REQUIREMENTS FOR WSDOT

Acceptance of Terms:

Acceptance of a state contract/purchase order by a Bidder/Supplier for any equipment purchased pursuant to this bid/contract constitutes acceptance of, and agreement with, all of the general and specific provisions, requirements, stipulations, and equipment specification(s) described in this bid/contract.

General:

Equipment offered for this bid/contract must be new (unused) and a current production model that requires no manufacturer or dealership modifications.  Units may be sold, prepared, and delivered to WSDOT, or its designated agent, by a dealer who is factory franchised for the specific makes and models of equipment offered.

All accessories and features listed herein shall be those supplied by the Original Equipment Manufacturer (OEM).  Any accessories, features, or operational performance required by FMVSS, Washington State Motor Vehicle Laws, OSHA,WISHA and Labor and Industries laws or mandates, that apply to the equipment being bid, shall be provided..  Additionally; all required certifications, inspections, and decaling shall be preformed prior to WSDOT acceptance.   All units associated with this purchase shall be of the same design and quality as those sold through normal retail channels; and they shall possess the latest technology, accessories, and features offered on standard retail units; whether or not they are called for in the following specifications.

Failure to comply with any of the specified requirements of the contract constitutes a breach of contract.  WSDOT may cancel all or any part of this contract, without incurring any costs whatsoever, including delivery, shipping, or re-stocking fees.

Specification Clarification and Changes:

Clarification for any item in these specifications may be obtained from the Department of Enterprise Services (DES), Office of State Procurement (OSP).

State contracts, for the purchase of state owned equipment, are official state documents that carry legal implications.  After award, there shall be no deviations from any requirement stated in the contract during the manufacturing or assembly process of the equipment identified herein, without a contract change order issued by DES, OSP.

Equipment Demonstration:

and During the bid evaluation process and prior to a contract award, Bidder(s) may be required to demonstrate the performance capabilities of the equipment offered in their bid.

Performance demonstration(s) must be conducted within fourteen (14) calendar days after notification that such demonstration(s) is required.  Bidder(s) shall conduct all demonstration(s) within Washington State and are responsible for their costs associated with the demonstration(s).

The Bidder(s) will coordinate with WSDOT, Equipment Office at (360) 705-7897/7884 to establish the location, date, and time of the performance demonstration(s).

Delivery; and Acceptance:

WSDOT will require the successful vendor to provide the “Supplier Confirmation of Specification Compliance” at time of delivery.  The form is attached to this document.  Suppliers are responsible for ensuring that all equipment purchased, pursuant to this contract, complies with all of the requirements and specifications listed in the contract.

Supplier(s) shall provide the following documents for review at time of delivery for each item:

- The manufacturer’s line production sheet stating the equipment serial number(s) and listing all of the equipment’s components;

- Completed Invoice;

- Supplier Confirmation of Specification Compliance;

- The Manufacturer’s Statement of Origin (MSO);

- Axle weight slips (for all units with axles); and

- A completed Washington State title application showing both the legal and registered owner as:  Washington State Department of Transportation, 7345 Linderson Way S.W. Tumwater WA 98501.  The mailing address is: P.O. Box 47357, Olympia WA 98504. Out of state Supplier(s) may contact WSDOT, Equipment Office at 360-705-7882/7884 to obtain a Washington State title application.

Suppliers must notify WSDOT at the phone number listed on the contract or the equipment order, twenty-four (24) hours prior to equipment delivery.  This is to ensure that a WSDOT employee is available to sign and date the bill of laden (or other delivery document) and receive the above-mentioned documents to indicate WSDOT has accepted delivery of the equipment.

When the unit is ready for final delivery, it may be delivered to WSDOT facilities (as specified on the contract and/or equipment order) between the hours of 7:00 am and 3:00 pm, Monday through Friday. Deliveries shall not be made during other hours, on weekends, or on legally recognized state and federal holidays.  WSDOT will not accept any responsibility for equipment that has been delivered to or left at a WSDOT facility unless a WSDOT employee has signed and dated the bill of laden or other delivery documents indicating WSDOT has accepted delivery of the equipment.

Supplier shall be solely liable for any equipment damages that occurred prior to WSDOT accepting delivery of the equipment.

Supplier Shall Meet the Delivery Terms of this Contract:

Should the Supplier become aware, in advance of an equipment delivery date, that the Supplier will be unable to meet the contract delivery date, the Supplier may make a written request to DES, OSP for a contract change order modifying the equipment delivery date.  Should the Supplier fail to meet the contract equipment delivery date, liquidated damages will be assessed.  The amount of liquidated damages will be calculated by using WSDOT’s established Equipment Rental Rate Schedule that is in effect as of the date of the contract or equipment order.  Liquidated damages will be assessed at the established per day equipment rental rate for each late delivery day, not to exceed ten (10) percent of the equipment’s purchase price.  This assessment represents a reasonable forecast of WSDOT’s actual damages for having to rent replacement equipment.  WSDOT shall deduct the liquidated damages for late delivery from the Supplier’s invoice.

Warranty Services and Performance:

Equipment suppliers must provide technical support and reasonable equipment modifications for a period of 90 calendar days after the date the equipment is reported in-ervice per manufacturer and/or factory warranty requirements. (??) This is to ensure that the purchased equipment is capable of performing the specified operational functions.

Bidders/Suppliers must include, as part of the bid, the factory and/or manufacturer’s one year warranty, which shall cover one hundred (100) percent parts and labor for the entire unit offered. This warranty must be honored by all authorized factory and/or manufacturer’s dealerships.

Supplier shall be liable for all costs associated with warranty repair(s), including, but not limited to, materials, parts, labor, and transport of equipment that are disabled due to the failure of the equipment during the warranty period.

Warranty coverage will not commence until the date the completed equipment is placed into service as reported by WSDOT pursuant to the warranty requirements or 30 days after final payment for the equipment, whichever occurs first.

During the warranty period Supplier must begin physical repairs on equipment failures within seventy-two (72) hours after WSDOT has notified the Supplier of an equipment failure.  Should the Supplier fail to begin equipment repairs within 72 hours after notification, WSDOT may elect (based on operational requirements) to make the warranty repairs.  Should WSDOT elect to make such warranty repairs, the Supplier agrees to fully reimburse WSDOT for all parts, materials, labor, shipping, and travel costs incurred by WSDOT for such warranty repairs.   WSDOT shall provide Supplier with a detailed invoice; and Supplier agrees to remit payment to WSDOT within thirty (30) days after receipt of the invoice.  Failure to provide and complete warranty service/repair within two (2) weeks (fourteen [14] calendar days) of the vehicle/equipment entering contractor’s facility may be sufficient grounds for the State to deny the contractor from bidding in future contracts due to poor performance.

Contractor shall provide a loaner (of similar type) in the event warranty repairs exceed a period of five (5) days.  This loaner shall be provided at no additional cost to the State.

During warranty period, the Supplier may, upon notification of a warranty failure, authorize WSDOT equipment repair technicians to make warranty repairs when it is advantageous to WSDOT and the Supplier. The Supplier shall reimburse WSDOT for all costs associated with the warranty repair.

Training:

Equipment Supplier shall provide on-site instructor(s) to conduct eight (8) hours of operator training per unit delivered and eight (8) hours of repair technician training per unit delivered.  During the eight hour training period, the length and number of training session(s) required may vary based on the equipment’s complexity and personnel experience levels.  Training session(s) may be less than eight hours should the WSDOT on-site supervisor or Equipment Training Manager determine that all personnel have completed training and the Supplier’s training obligation has been fulfilled. The training session(s) shall include, but not be limited to, the below listed items.

- Operator training will be designed to familiarize personnel with the controls, safety features, operating characteristics, and operator checks and services.

- Operator training may include teaching operators shifting, acceleration, and braking techniques to maximize operational effectiveness of the unit's power train configuration for equipment so configured.

- Mechanic training shall be designed to familiarize service and repair technicians with preventive maintenance checks and services, system diagnostics procedures, repairs, adjustments, and any unique requirements associated with the unit.

All training shall be scheduled and coordinated with the ship to addressee.  Coordination will include dates, times, location, number of students per session, number of sessions required, facilities, and training equipment and material.

Qualified individuals shall conduct training sessions.  "Qualified" means that the trainer must have a high level of knowledge and experience relating to the type of equipment offered or purchased:

- Person(s) conducting the operator training session(s) must have a minimum of one (1) year of experience actually operating the unit for which training is being conducted or be a factory/manufacturer certified trainer.

- Person(s) conducting repair technician training session(s) must have at least one (1) year of experience in the performance of preventive maintenance and repair on the unit for which the training is being conducted or be a factory/manufacturer certified trainer.

An on-site WSDOT supervisor or the Equipment Training Manager will evaluate the training sessions and will determine whether or not the training is adequate.  If the training is deemed inadequate, the Supplier agrees to conduct additional training sessions, at no cost to WSDOT, to the satisfaction of WSDOT.

Special Notes/Workmanship:

COVERS: All caps and covers that must be removed in order to perform daily to bi-weekly preventive maintenance, scheduled warranty service, or maintenance recommended or required by the Contractor, must be secured to the unit by a chain or other device. All hoses, pipes, and plumbing connections shall have either retained covers, quick disconnects, or protective caps.

HOSE, WIRE AND TUBE ROUTING: Hose, wire and tube routing shall not impede normal maintenance and adjustment of the unit.  Hoses, wires and tubes shall be securely and neatly positioned.  Kinks in hoses, wires, or tubing are not acceptable.

HYDRAULIC SYSTEMS (Workmanship Standards applicable to both traction and controls systems):

Components containing hydraulic fluid shall not be installed in the cab.

Hydraulic Symbols: Hydraulic symbols on schematics shall be interpreted per American National Standard Institute (ANSI) Y32.10.

Leaks: Hydraulic leaks are not acceptable.  A leak is defined as any fluid flow larger than one (1) drop in four (4) hours.

All components installed in the hydraulic system shall be free of contamination and shall be flushed out or cleaned, if necessary, to meet this requirement.

Hose Routing: Hose routing shall meet the following requirements:

    - The bend radii of hoses shall be not be less than the manufacturers' recommended minimums.

    - Hoses shall be routed no closer than six (6) inches from exhaust components or other heat sources unless proper shielding is provided.

    - Hoses shall not come in contact with moving parts.

    - Hoses shall be clamped to supporting structures at intervals not exceeding sixty (60) inch hose diameters.  Clamps shall be Beranger style or equal cushion clamps.

    - Hydraulic hoses will NOT be bundled together using wire ties.  This shall be accomplished using the Beranger or equal clamps.

    - Hoses shall not be routed through holes or across sharp edges without protection from being chafed or cut.

      - Hoses shall be of sufficient length to prevent stretching, distortion, and disconnecting.

Pipe fittings shall not be used on the pressure side of any hydraulic circuit.  A Teflon sealant shall be used on pipe threads.  Sealant shall be used sparingly and shall not contaminate the hydraulic system.

All hydraulic components shall be rated for working pressures to exceed system operating pressures, and have a minimum burst of one and one half (1-1/2) times the system operating pressure.

All hydraulic quick disconnects shall be equipped with protective and retained covers

ELECTRICAL WORKMANSHIP AND MATERIALS:

All wire shall be un-tinned stranded copper with cross-linked polyethylene insulation.  Wires shall be color coded.  Wire runs and color coding shall be continuous and terminate within a weatherproof junction box or weather pack connectors.

Unless otherwise specified and except for multi-terminal connectors, solderless terminals shall be used for all connections.  Solder all ring terminals on ten (10) AWG and smaller wires shall be insulated.

Insulated ring terminals shall meet military specification.  Military spec ring terminals have an extra copper sleeve around the terminal barrel that improves the holding force and reduces corrosion.

Butt splices shall have moisture barriers and integral polyolefin heat shrinkable sleeves.  Heat shrink tubing over non-sealed butt splices is not acceptable.

Insulation-piercing connectors (scotch locks) shall not be used.

Wires shall be protected by looms, sleeving, and grommets.  Wire bundles shall be routed away from moving parts and hot components.  Wires shall not be bundled with any electrical wiring or components.

All chassis/body combination units shall have a separate grounding strap installed from the body to the chassis.

Wire routing shall ensure that the following requirements are met:

- Wires shall not be routed closer than six (6) inches to exhaust components or other heat sources, unless they are shielded.

- Wires shall not come in contact with moving parts.

Wires and wire harnesses shall not be routed through holes or across sharp edges without protection from being chafed or cut.

GENERAL WELDING REQUIREMENTS:

Distortion of assembled parts is not acceptable.  All welds will have proper penetration and be relatively uniform in appearance.  All welds shall provide a metal-to-metal bond using proper flux and/or welding materials.  Continuous welds shall be used wherever possible and practical.

Body Fillers: The covering of welds with body fillers or similar practice is not acceptable.

OPERATOR(S) - EQUIPMENT INTERFACE:

Control Locations:  The operator(s) stations(s), including safety devices, controls, and gauges, shall be accessible, readable, and visible to the operator without distracting from the safe operational requirements of the equipment ordered.

Vehicle Delivery Checklist   (provide at time of delivery if awarded)

| Supplier Specification Confirmation | Supplier Initials and Date |
| --- | --- |
| The supplier will ensure all of the annotated items listed below are inspected prior to delivery. |  |
| Supplier is 100 % compliant with the bid specification(s) as bid by the supplier. |  |
| The unit is clean inside/outside upon delivery to WSDOT |  |
| All required documentation as per the WSDOT and OSP bidding requirements are presented at time of delivery. Example: |  |
| - MSO |  |
| - Title Application |  |
| - Axle weight slip (if required) |  |
| - Supplier Confirmation of Specification Compliance (this form) |  |
| - Invoice |  |
| All safety equipment is provided and in proper working condition. (Lights, reflectors, seatbelts, etc.) |  |
| All required publications are brought with the unit upon delivery. |  |
| Workmanship Issues: |  |
| - All hoses and wiring are protected from damage. |  |
| - All fluid levels have been checked and are at the manufacturer’s suggested levels. |  |
| - All components are in proper working condition. |  |
| - Welds are consistent and without cracks |  |
| - No runs, cracks or chips in the paint. |  |
| Printed Vendor Name _____________________________ |  |
| Vendor Signature _____________________________ | Date________ |

C.3 TECHNICAL SPECIFICATIONS

Category 1:  3 Ton, Vibratory, Steel Wheeled Roller

|  | # Vibratory; Steel Wheeled; 3 Ton |
| --- | --- |

| Specification Requirements |  | Check If Meet or Exceed |  | Describe Offered Alternatives Note: Alternatives offered will be evaluated by subject matter experts. Alternatives offered that are not found to meet the specification requirements may cause the bid to be rejected. |
| --- | --- | --- | --- | --- |
| - General: |  |  |  |  |
| - Shall be a Model CB24 or equal |  |  |  |  |
| - This specification describes the requirements for a 6,000-pound Static Steel Wheeled Roller. |  |  |  |  |
| - Shall have an operating weight ranging of 5,700 to 6,900 pounds. |  |  |  |  |
| - The unit shall be an articulated frame type compactor. |  |  |  |  |
| - The front and rear frame shall allow at least +6 degrees of oscillation from horizontal. |  |  |  |  |
| - The successful bidder will be required deliver the completed unit to one of the delivery locations annotated below. |  |  |  |  |
| - 6431 Corson Ave. Seattle Wa. 98108 - 2830 Euclid Ave. Bldg. D Wenatchee Wa. 98807 - 5720 Capitol Blvd. Tumwater Wa. 98501 - 4200 Main St. Vancouver Wa. 98668 - 2809 Rudkin Rd. Union Gap Wa. 98909-2560 - 221 E. North Foothills Dr. Spokane Wa. 99207-2090 |  |  |  |  |
| - Engine: |  |  |  |  |
| - Shall be the manufacturer’s standard direct injection diesel engine. |  |  |  |  |
| - Shall have a 2-stage dry type air cleaner with restriction indicator. |  |  |  |  |
| - Shall be equipped with a 12 or 24 volt electrical system. |  |  |  |  |
| - If equipped with a vertical exhaust pipe, it shall be outfitted with a full-length heat shroud. |  |  |  |  |
| - Transmission: |  |  |  |  |
| - Shall be a closed circuit infinitely variable Hydrostatic drive system, operated by a single lever control for the direction and speed. |  |  |  |  |
| - Shall be equipped with both forward and reverse. |  |  |  |  |
| - Shall have a neutral safety switch. |  |  |  |  |
| - The hydraulic propel lever's two step regulating curve shall provide smooth and precise control at low speed |  |  |  |  |
| - Hydraulics: |  |  |  |  |
| - Shall be equipped with a hydraulic system, which is compatible with the propulsion, and steering systems. |  |  |  |  |
| - The Hydraulic connections shall include O-ring face seal design |  |  |  |  |
| - Shall be equipped with a 10-micron filter on the suction line. |  |  |  |  |
| - All hydraulic hoses shall be separated and secured to prevent abrasion. |  |  |  |  |
| - Drums: |  |  |  |  |
| - The drums shall be at least 27 inches in diameter and 47 inches in width. |  |  |  |  |
| - Drums shall have rounded edges and machined rolling surfaces. |  |  |  |  |
| - Each drum shall be equipped with one spring tensioned, flexible wiping device mounted on the frame structure which surrounds the drum. |  |  |  |  |
| - Water Spray System: |  |  |  |  |
| - Shall have a pressurized no-corrosive system with minimum 35 gallon capacity. |  |  |  |  |
| - Shall be equipped with a filler cap, screen and drain plug and water level indicator. |  |  |  |  |
| - Shall be equipped with an electrical pump to provide pressurized water to the fan spray nozzles. The pump shall have the capability of being run dry with no damage to the pump. |  |  |  |  |
| - The system shall have in-line filtration. |  |  |  |  |
| - Spray system control shall consist of a 3-position water spray switch with 3 modes, constant on or pulsating, off, and automatic on and off or pulsating corresponding to the machines travel and water flow valve. |  |  |  |  |
| - The standard pressurized water spray system shall be capable of constant or intermittent spray |  |  |  |  |
| - Steering: |  |  |  |  |
| - Shall be hydrostatic, full power, and articulated. |  |  |  |  |
| - The turning radius shall not exceed 153 inches outside and 111 inches inside. |  |  |  |  |
| - Brakes: |  |  |  |  |
| - Service brakes shall be through the hydrostatic transmission. |  |  |  |  |
| - The parking/emergency brake shall be a spring applied, hydraulically released, disc type. |  |  |  |  |
| - The parking/emergency brake shall have the ability to automatically apply when hydraulic pressure is lost. |  |  |  |  |
| - Electrical System: |  |  |  |  |
| - Shall have a 12-volt DC system with a maintenance free battery. |  |  |  |  |
| - Shall be equipped with minimum 60-Ampere alternator. |  |  |  |  |
| - Lights: |  |  |  |  |
| - Shall have dual working/headlights front and rear. |  |  |  |  |
| - Shall be equipped with front and rear four way flashers and turn signals. |  |  |  |  |
| - Shall have Warning Rotator light mounted on ROPS |  |  |  |  |
| - ROPS: |  |  |  |  |
| - Shall have a rollover protection structure with seat belts. |  |  |  |  |
| - The structure shall not obstruct the operator’s view or over hang the frame. |  |  |  |  |
| - Operator’s Station: |  |  |  |  |
| - The control console shall be the manufacturer standard to include the following. |  |  |  |  |
| - The operator’s platform shall have a non-slip walkway, with reinforced railing, hand holds |  |  |  |  |
| - Unit shall be equipped with a lockable instrument panel with the following monitoring system. WSDOT will accept warning lights. |  |  |  |  |
| - Fuel gauge |  |  |  |  |
| - Water temperature |  |  |  |  |
| - Oil pressure |  |  |  |  |
| - Hour meter |  |  |  |  |
| - Volt Meter |  |  |  |  |
| - Parking brake warning light. |  |  |  |  |
| - Air Filter restriction indicator. |  |  |  |  |
| - All lockable compartments shall be keyed alike. |  |  |  |  |
| - Exterior Finish: |  |  |  |  |
| WSDOT expects professional workmanship on all products purchased. With this in mind, the following finishing requirements will be closely scrutinized during the specification compliance inspection. |  |  |  |  |
| - There shall be no welding scale, roughness, sharp corners; or rust stains on the unit. |  |  |  |  |
| - The unit shall be coated with 2 mils dry automotive quality primers, with an additional 2 mils dry of the manufacturer’s standard color, finish paint. |  |  |  |  |
| - The body paint shall be warranted against rust and corrosion for five years. |  |  |  |  |
| 4. The unit supplier shall contact Billboard Signs Company, at 206-300-7300, to have the unit decaled prior to the WSDOT specification compliance inspection. Billboard Signs will invoice WSDOT directly for all unit decaling. |  |  |  |  |
| - Publications: |  |  |  |  |
| - The unit shall be delivered with an operator’s manual. |  |  |  |  |
| - Bidders shall provide the Service, and Parts Manuals as annotated below. 1 Parts Manual or CD (Per Unit) 1 Service and technical Manual or CD (Per Unit) |  |  |  |  |

Category 2:  4 Ton, Vibratory, Steel Wheeled Roller

|  | # Vibratory; 4 Ton |
| --- | --- |

| Specification Requirements |  | Check If Meet or Exceed |  | Describe Offered Alternatives Note: Alternatives offered will be evaluated by subject matter experts. Alternatives offered that are not found to meet the specification requirements may cause the bid to be rejected. |
| --- | --- | --- | --- | --- |
| - General: |  |  |  |  |
| - Shall be a Dynapac Model CC142 or equal |  |  |  |  |
| - This specification describes the requirements for an 8,400-pound Vibratory Steel Wheeled Roller. |  |  |  |  |
| - The unit shall be an articulated frame type compactor. |  |  |  |  |
| - The front and rear frame shall allow +6 degrees of oscillation from horizontal. |  |  |  |  |
| 5. The successful bidder will be required deliver the completed unit to one of the delivery locations annotated below. |  |  |  |  |
| - 6431 Corson Ave. Seattle Wa. 98108 - 2830 Euclid Ave. Bldg. D Wenatchee Wa. 98807 - 5720 Capitol Blvd. Tumwater Wa. 98501 - 4200 Main St. Vancouver Wa. 98668 - 2809 Rudkin Rd. Union Gap Wa. 98909-2560 - 221 E. North Foothills Dr. Spokane Wa. 99207-2090 |  |  |  |  |
| - Engine: |  |  |  |  |
| - Shall be the manufacturer’s standard direct injection diesel engine. |  |  |  |  |
| - Shall have a 2-stage dry type air cleaner with restriction indicator. |  |  |  |  |
| - Shall be equipped with a 12 or 24-volt electrical system. |  |  |  |  |
| - Transmission: |  |  |  |  |
| - Shall be a closed circuit infinitely variable Hydrostatic drive system, operated by a single lever control for the direction and speed. |  |  |  |  |
| - Shall be equipped with both forward and reverse. |  |  |  |  |
| - Shall have a neutral safety switch. |  |  |  |  |
| - Hydraulics: |  |  |  |  |
| - Shall be equipped with a hydraulic system, which is compatible with the propulsion, vibration and steering systems. |  |  |  |  |
| - Shall be equipped with a 10-micron filter on the suction line. |  |  |  |  |
| - If equipped with a vertical exhaust pipe, it shall be outfitted with a full-length heat shroud. |  |  |  |  |
| - Vibratory System: |  |  |  |  |
| - Shall be equipped with the manufacturer’s standard hydraulic system. |  |  |  |  |
| - The hydraulic motor shall be directly coupled to the drums eccentric shaft vibrator in each drum. |  |  |  |  |
| - The unit shall have a vibration frequency of at least 3000 vpm. |  |  |  |  |
| - Each vibrator shall have the ability for manual and or automatic cutout for each of the vibrators. |  |  |  |  |
| - Drums: |  |  |  |  |
| - The drums shall be at least 29 inches in diameter of at least 50 inches in width. |  |  |  |  |
| - Drums shall have rounded edges and machined rolling surfaces. |  |  |  |  |
| - Each drum shall be equipped with one spring tensioned, flexible wiping device mounted on the frame structure which surrounds the drum. |  |  |  |  |
| - The drums and their vibrating parts shall be isolated from the rest of the roller by replaceable supports, which isolate the vibration from the operator. |  |  |  |  |
| - Water Spray System: |  |  |  |  |
| - Shall have a pressurized no-corrosive system with 50 gallons of water. |  |  |  |  |
| - Shall be equipped with a filler cap, screen and drain plug. |  |  |  |  |
| - Shall be equipped with an electrical pump to provide pressurized water to the fan spray nozzles. The pump shall have the capability of being run dry with no damage to the pump. |  |  |  |  |
| - The system shall have in-line filtration. |  |  |  |  |
| - Spray system control shall consist of a 3-position water spray switch with 3 modes, constant on or pulsating, off, and automatic on and off or pulsating corresponding to the machines travel and water flow valve. |  |  |  |  |
| - Steering: |  |  |  |  |
| - Shall be hydrostatic, full power, and articulated. |  |  |  |  |
| - The inside turning radius shall not exceed 118 inches. |  |  |  |  |
| - Brakes: |  |  |  |  |
| - The parking/emergency brake shall be a spring applied, hydraulically released, disc type. |  |  |  |  |
| - The parking/emergency brake shall have the ability to automatically apply when hydraulic pressure is lost. |  |  |  |  |
| - Electrical System: |  |  |  |  |
| - Shall have a 12-24 volt DC system with maintenance free battery (s). |  |  |  |  |
| - Shall be equipped with an alternator. |  |  |  |  |
| - Lights: |  |  |  |  |
| Shall have dual working/headlights front and rear. |  |  |  |  |
| - ROPS: |  |  |  |  |
| - Shall have a rollover protection structure with seat belts. |  |  |  |  |
| - The structure shall not obstruct the operator’s view or over hang the frame. |  |  |  |  |
| - Operator’s Station: |  |  |  |  |
| - The control console shall be the manufacturer standard to include the following. |  |  |  |  |
| - The operator’s platform shall be equipped with a vibration isolation system. |  |  |  |  |
| - The operator’s platform shall have a non-slip walkway, with reinforced railing and hand holds. |  |  |  |  |
| - Unit shall be equipped with a lockable instrument panel with the following monitoring system. WSDOT will accept warning lights. |  |  |  |  |
| - Fuel gauge |  |  |  |  |
| - Water temperature gauge |  |  |  |  |
| - Oil pressure |  |  |  |  |
| - Hour meter |  |  |  |  |
| - Volt Meter |  |  |  |  |
| - Parking brake warning light. |  |  |  |  |
| - Air Filter restriction indicator. |  |  |  |  |
| - Unit shall be equipped with a lockable instrument panel to include the hood. |  |  |  |  |
| - All lockable compartments shall be keyed alike. |  |  |  |  |
| - Exterior Finish: |  |  |  |  |
| WSDOT expects professional workmanship on all products purchased. With this in mind, the following finishing requirements will be closely scrutinized during the specification compliance inspection. |  |  |  |  |
| 1. There shall be no welding scale, roughness, sharp corners; or rust stains on the unit. |  |  |  |  |
| 2. The unit shall be coated with 2 mils dry automotive quality primers, with an additional 2 mils dry of the manufacturers’ standard color, finish paint. |  |  |  |  |
| 3. The body paint shall be warranted against rust and corrosion for five years. |  |  |  |  |
| 4. The unit supplier shall contact Billboard Signs Company, at 206-300-7300, to have the unit decaled prior to the WSDOT specification compliance inspection. Billboard Signs will invoice WSDOT directly for all unit decaling. |  |  |  |  |
| - Publications: |  |  |  |  |
| - The unit shall be delivered with an operator’s manual. |  |  |  |  |
| - Bidders shall provide the Service, and Parts Manuals as annotated below. 1 Parts Manual or CD (Per Unit) 1 Service Manual or CD (Per Unit) |  |  |  |  |

Category 3:  27,000 – Pound Rubber Wheeled Roller

|  | # Self-Propelled; Rubber Tires |
| --- | --- |

| Specification Requirements |  | Check If Meet or Exceed |  | Describe Offered Alternatives Note: Alternatives offered will be evaluated by subject matter experts. Alternatives offered that are not found to meet the specification requirements may cause the bid to be rejected. |
| --- | --- | --- | --- | --- |
| - General: |  |  |  |  |
| - This specification describes the requirements for a 27,000,000-pound Rubber Tired Roller. |  |  |  |  |
| - The roller shall be capable of 27,000 pounds of operating weight. |  |  |  |  |
| - Shall be equipped with at least one inch thick scraper pads over each tire (Front and Rear) |  |  |  |  |
| - The unit shall have oscillating suspension both front and rear. |  |  |  |  |
| 5. The overall height of the Roller shall not exceed 127 inches to include the ROPS system. |  |  |  |  |
| 6. The successful bidder will be required deliver the completed unit to one of the delivery locations annotated below. |  |  |  |  |
| - 6431 Corson Ave. Seattle Wa. 98108 - 2830 Euclid Ave. Bldg. D Wenatchee Wa. 98807 - 5720 Capitol Blvd. Tumwater Wa. 98501 - 4200 Main St. Vancouver Wa. 98668 - 2809 Rudkin Rd. Union Gap Wa. 98909-2560 - 221 E. North Foothills Dr. Spokane Wa. 99207-2090 |  |  |  |  |
| - Engine: |  |  |  |  |
| - Shall be a liquid cooled diesel capable of at least 84 horsepower. |  |  |  |  |
| - Shall have a 2-stage dry type air cleaner with restriction indicator. |  |  |  |  |
| - Coolant system shall be protected to -35 deg. |  |  |  |  |
| - Shall be equipped with a 12 or 24volt electrical system. |  |  |  |  |
| - Transmission: |  |  |  |  |
| Shall be a Hydraulic drive system. |  |  |  |  |
| - Hydraulics: |  |  |  |  |
| - Shall be equipped with a hydraulic system, which is compatible with the propulsion, and steering systems. |  |  |  |  |
| - Shall be equipped with a 10-micron filter on the suction line. |  |  |  |  |
| - Water Spray System: |  |  |  |  |
| - Shall have a non -corrosive system with not less than 75 gallons of water. |  |  |  |  |
| - The reservoir (s) shall be equipped with a filler cap, screen and drain plug. |  |  |  |  |
| - Shall be equipped with an electrical pump to provide pressurized water to the fan spray nozzles. The pump shall have the capability of being run dry with no damage to the pump. |  |  |  |  |
| - The system shall have in-line filtration. |  |  |  |  |
| - Spray system control shall consist of a 3-position water spray switch with 3 modes, constant on or pulsating, off, and automatic on and off or pulsating corresponding to the machines travel and water flow valve. |  |  |  |  |
| - Steering: |  |  |  |  |
| Shall be hydraulic with full power. |  |  |  |  |
| - Brakes: |  |  |  |  |
| - The parking/emergency brake shall be a spring applied, hydraulically released, disc type. |  |  |  |  |
| - The parking/emergency brake shall have the ability to automatically apply when hydraulic pressure is lost. |  |  |  |  |
| - Electrical System: |  |  |  |  |
| - Shall have a 12-or 24 volt DC system with a maintenance free battery. |  |  |  |  |
| - Shall be equipped with an Alternator. |  |  |  |  |
| - Lights: |  |  |  |  |
| Shall have dual working/headlights front and rear. |  |  |  |  |
| - ROPS: |  |  |  |  |
| - Shall have a falling object/rollover protection structure with seat belts. |  |  |  |  |
| - The structure shall not obstruct the operator’s view or over hang the frame. |  |  |  |  |
| - Operator’s Station: |  |  |  |  |
| - The roller shall have the ability to operate in either direction without having to turn the unit around. Note: Describe how this is accomplished. WSDOT reserves the right to determine if this explanation is satisfactory in order to determine if the bid is responsive to this requirement. ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________ |  |  |  |  |
| - The operator’s platform shall have a non-slip walkway, with reinforced railing and hand holds. |  |  |  |  |
| - Unit shall be equipped with a lockable instrument panel with the following monitoring system. WSDOT will accept warning lights. |  |  |  |  |
| - Fuel gauge. |  |  |  |  |
| - Water temperature gauge. |  |  |  |  |
| - Oil pressure. |  |  |  |  |
| - Hydrostatic temperature gauges. |  |  |  |  |
| - Hour meter. |  |  |  |  |
| - Volt Meter. |  |  |  |  |
| - Parking brake warning light. |  |  |  |  |
| - Spray system water level indicator. |  |  |  |  |
| - No water in spray system warning lights. |  |  |  |  |
| - Unit shall be equipped with a lockable instrument panel to include the hood. |  |  |  |  |
| - Exterior Finish: |  |  |  |  |
| WSDOT expects professional workmanship on all products purchased. With this in mind, the following finishing requirements will be closely scrutinized during the specification compliance inspection. |  |  |  |  |
| 1. There shall be no welding scale, roughness, sharp corners; or rust stains on the unit. |  |  |  |  |
| 2. The unit shall be coated with 2 mils dry automotive quality primers, with an additional 2 mils dry of the manufacturers standard color, finish paint. |  |  |  |  |
| 3. The body paint shall be warranted against rust and corrosion for five years. |  |  |  |  |
| 4. The unit supplier shall contact Billboard Signs Company, at 206-300-7300, to have the unit decaled prior to the WSDOT specification compliance inspection. Billboard Signs will invoice WSDOT directly for all unit decaling. |  |  |  |  |
| - Publications: |  |  |  |  |
| - The unit shall be delivered with an operator’s manual. |  |  |  |  |
| - Bidders shall provide the Service, and Parts Manuals as annotated below. 1 Parts Manual or CD (Per Unit) 1 Service Manual or CD (Per Unit) |  |  |  |  |

Category 4:  3 Ton, Vibratory, Steel Wheeled Roller

| 1401 | # Static; Steel Wheeled; 4 Ton |
| --- | --- |

| Specification Requirements |  | Check If Meet or Exceed |  | Describe Offered Alternatives |
| --- | --- | --- | --- | --- |
| - General: |  |  |  |  |
| - Shall be a Model CB 22 or equal |  |  |  |  |
| - This specification describes the requirements for a 6,000-pound Static Steel Wheeled Roller. |  |  |  |  |
| - Shall have an operating weight ranging of 5,730 to 6,950 pounds. |  | X |  | 5,512 |
| - The unit shall be an articulated frame type compactor. |  |  |  |  |
| - The front and rear frame shall allow +10 degrees of oscillation from horizontal. |  |  |  |  |
| - Engine: |  |  |  |  |
| - Shall be a water-cooled, 3 cylinders direct injection diesel engine, developing 28 horsepower. |  |  |  |  |
| - Shall have a 2-stage dry type air cleaner with restriction indicator. |  |  |  |  |
| - Shall be equipped with a 12-volt electrical system. |  |  |  |  |
| - Transmission: |  |  |  |  |
| - Shall be a closed circuit infinitely variable Hydrostatic drive system, operated by a single lever control for the direction and speed. |  |  |  |  |
| - Shall be equipped with both forward and reverse. |  |  |  |  |
| - Shall have a neutral safety switch. |  |  |  |  |
| - Hydraulics: |  |  |  |  |
| - Shall be equipped with a hydraulic system, which is compatible with the propulsion, and steering systems. |  |  |  |  |
| - Shall be equipped with a 10-micron filter on the suction line. |  |  |  |  |
| - If equipped with a vertical exhaust pipe, it shall be outfitted with a full-length heat shroud. |  | N/A |  |  |
| - Drums: |  |  |  |  |
| - The drums shall be 27 inches in diameter and 38 to 40 inches in width. |  |  |  | 39” |
| - Drums shall have rounded edges and machined rolling surfaces. |  |  |  |  |
| - Each drum shall be equipped with one spring tensioned, flexible wiping device mounted on the frame structure which surrounds the drum. |  |  |  |  |
| - Water Spray System: |  |  |  |  |
| - Shall have a pressurized no-corrosive system with 42 gallons of water. |  | X |  | 39.6 |
| - Shall be equipped with a filler cap, screen and drain plug and water level indicator. |  |  |  |  |
| - Shall be equipped with an electrical pump to provide pressurized water to the fan spray nozzles. The pump shall have the capability of being run dry with no damage to the pump. |  |  |  |  |
| - The system shall have in-line filtration. |  |  |  |  |
| - Spray system control shall consist of a 3-position water spray switch with 3 modes, constant on, off, and automatic on and off corresponding to the machines travel and water flow valve. |  |  |  | PULSATING OR INTERMITTENT |
| - Steering: |  |  |  |  |
| - Shall be hydrostatic, full power, and articulated. |  |  |  |  |
| - The turning radius shall not exceed 150 inches outside and 102 inches inside. |  | X |  | 143” OUT & 104” IN |
| - Brakes: |  |  |  |  |
| - Service brakes shall be through the hydrostatic transmission. |  |  |  |  |
| - The parking/emergency brake shall be a spring applied, hydraulically released, disc type. |  |  |  |  |
| - The parking/emergency brake shall have the ability to automatically apply when hydraulic pressure is lost. |  |  |  |  |
| - Electrical System: |  |  |  |  |
| - Shall have a 12-volt DC system with a maintenance free battery. |  |  |  |  |
| - Shall be equipped with a 60-Ampere alternator. |  |  |  |  |
| - Lights: |  |  |  |  |
| - Shall have dual working/headlights front and rear. |  |  |  |  |
| - Shall be equipped with front and rear four way flashers and turn signals. |  |  |  |  |
| - ROPS: |  |  |  |  |
| - Shall have a rollover protection structure with seat belts. |  |  |  |  |
| - The structure shall not obstruct the operator’s view or over hang the frame. |  |  |  |  |
| - Operator’s Station: |  |  |  |  |
| - The control console shall be the manufacturer standard to include the following. |  |  |  |  |
| - The operator’s platform shall have a non-slip walkway, with reinforced railing, hand holds |  |  |  |  |
| - Unit shall be equipped with a lockable instrument panel with the following monitoring system. |  |  |  |  |
| - Fuel gauge |  |  |  |  |
| - Water temperature |  |  |  |  |
| - Oil pressure |  |  |  |  |
| - Hour meter |  |  |  |  |
| - Volt Meter |  |  |  |  |
| - Parking brake warning light. |  |  |  |  |
| - Air Filter restriction indicator. |  |  |  |  |
| - All lockable compartments shall be keyed alike. |  |  |  |  |
| - Exterior Finish: |  |  |  |  |
| WSDOT expects professional workmanship on all products purchased. With this in mind, the following finishing requirements will be closely scrutinized during the specification compliance inspection. |  |  |  |  |
| - There shall be no welding scale, roughness, sharp corners; or rust stains on the unit. |  |  |  |  |
| - The unit shall be coated with 2 mils dry automotive quality primers, with an additional 2 mils dry of the manufacturers standard color, finish paint. |  |  |  |  |
| - The body paint shall be warranted against rust and corrosion for five years. |  |  |  |  |
| - The unit supplier shall contact Billboard Signs Company, at 206-300-7300, to have the unit decaled prior to the WSDOT specification compliance inspection. Billboard Signs will invoice WSDOT directly for all unit decaling. |  |  |  |  |
| - Publications: |  |  |  |  |
| - The unit shall be delivered with an operator’s manual. |  |  |  |  |
| - Bidders shall provide the Service, and Parts Manuals as annotated below. 1 Parts Manual (Per Unit) 1 Service Manual (Per Unit) |  |  |  |  |

Category 5: 2 Ton, Vibratory, Steel Wheeled Roller

| 1401 | # Static; Steel Wheeled; 4 Ton |
| --- | --- |

| Specification Requirements |  | Check If Meet or Exceed |  | Describe Offered Alternatives |
| --- | --- | --- | --- | --- |
| - General: |  |  |  |  |
| - Shall be a Model CB 22 or equal |  | YES |  | BOMAG BW100ADM-5 |
| - This specification describes the requirements for a 6,000-pound Static Steel Wheeled Roller. |  | YES |  | VIBRATORY |
| - Shall have an operating weight ranging of 5,730 to 6,950 pounds. |  | YES |  | 3638LBS |
| - The unit shall be an articulated frame type compactor. |  | YES |  |  |
| - The front and rear frame shall allow +10 degrees of oscillation from horizontal. |  | EXCEPTION |  | 8 DEGREES |
| - Engine: |  |  |  |  |
| - Shall be a water-cooled, 3 cylinders direct injection diesel engine, developing 28 horsepower. |  | YES |  | 20.2HP |
| - Shall have a 2-stage dry type air cleaner with restriction indicator. |  | YES |  |  |
| - Shall be equipped with a 12-volt electrical system. |  | YES |  |  |
| - Transmission: |  |  |  |  |
| - Shall be a closed circuit infinitely variable Hydrostatic drive system, operated by a single lever control for the direction and speed. |  | YES |  |  |
| - Shall be equipped with both forward and reverse. |  | YES |  |  |
| - Shall have a neutral safety switch. |  | YES |  |  |
| - Hydraulics: |  |  |  |  |
| - Shall be equipped with a hydraulic system, which is compatible with the propulsion, and steering systems. |  | YES |  |  |
| - Shall be equipped with a 10-micron filter on the suction line. |  | YES |  |  |
| - If equipped with a vertical exhaust pipe, it shall be outfitted with a full-length heat shroud. |  | YES |  |  |
| - Drums: |  |  |  |  |
| - The drums shall be 27 inches in diameter and 38 to 40 inches in width. |  | YES |  | 22.8” DIAMETER AND 39.4” WIDE |
| - Drums shall have rounded edges and machined rolling surfaces. |  | YES |  |  |
| - Each drum shall be equipped with one spring tensioned, flexible wiping device mounted on the frame structure which surrounds the drum. |  | YES |  |  |
| - Water Spray System: |  |  |  |  |
| - Shall have a pressurized no-corrosive system with 42 gallons of water. |  | YES |  | 26.4 GALLONS |
| - Shall be equipped with a filler cap, screen and drain plug and water level indicator. |  | YES |  |  |
| - Shall be equipped with an electrical pump to provide pressurized water to the fan spray nozzles. The pump shall have the capability of being run dry with no damage to the pump. |  | YES |  |  |
| - The system shall have in-line filtration. |  | YES |  |  |
| - Spray system control shall consist of a 3-position water spray switch with 3 modes, constant on, off, and automatic on and off corresponding to the machines travel and water flow valve. |  | YES |  |  |
| - Steering: |  |  |  |  |
| - Shall be hydrostatic, full power, and articulated. |  | YES |  |  |
| - The turning radius shall not exceed 150 inches outside and 102 inches inside. |  | YES |  | 78” INNER AND 117.4” OUTER |
| - Brakes: |  |  |  |  |
| - Service brakes shall be through the hydrostatic transmission. |  | YES |  |  |
| - The parking/emergency brake shall be a spring applied, hydraulically released, disc type. |  | YES |  |  |
| - The parking/emergency brake shall have the ability to automatically apply when hydraulic pressure is lost. |  | YES |  |  |
| - Electrical System: |  |  |  |  |
| - Shall have a 12-volt DC system with a maintenance free battery. |  | YES |  |  |
| - Shall be equipped with a 60-Ampere alternator. |  | YES |  |  |
| - Lights: |  |  |  |  |
| - Shall have dual working/headlights front and rear. |  | YES |  |  |
| - Shall be equipped with front and rear four way flashers and turn signals. |  | YES |  |  |
| - ROPS: |  |  |  |  |
| - Shall have a rollover protection structure with seat belts. |  | YES |  |  |
| - The structure shall not obstruct the operator’s view or over hang the frame. |  | YES |  |  |
| - Operator’s Station: |  |  |  |  |
| - The control console shall be the manufacturer standard to include the following. |  | YES |  |  |
| - The operator’s platform shall have a non-slip walkway, with reinforced railing, hand holds |  | YES |  |  |
| - Unit shall be equipped with a lockable instrument panel with the following monitoring system. |  | YES |  |  |
| - Fuel gauge |  | YES |  |  |
| - Water temperature |  | YES |  |  |
| - Oil pressure |  | YES |  |  |
| - Hour meter |  | YES |  | UNDER HOOD |
| - Volt Meter |  | YES |  |  |
| - Parking brake warning light. |  | YES |  |  |
| - Air Filter restriction indicator. |  | YES |  | UNDER HOOD |
| - All lockable compartments shall be keyed alike. |  | YES |  |  |
| - Exterior Finish: |  |  |  |  |
| WSDOT expects professional workmanship on all products purchased. With this in mind, the following finishing requirements will be closely scrutinized during the specification compliance inspection. |  | YES |  |  |
| - There shall be no welding scale, roughness, sharp corners; or rust stains on the unit. |  | YES |  |  |
| - The unit shall be coated with 2 mils dry automotive quality primers, with an additional 2 mils dry of the manufacturers standard color, finish paint. |  | YES |  |  |
| - The body paint shall be warranted against rust and corrosion for five years. |  | EXCEPTION |  | ONE YEAR OR 1000 HOURS |
| - The unit supplier shall contact Billboard Signs Company, at 206-300-7300, to have the unit decaled prior to the WSDOT specification compliance inspection. Billboard Signs will invoice WSDOT directly for all unit decaling. |  | YES |  |  |
| - Publications: |  |  |  |  |
| - The unit shall be delivered with an operator’s manual. |  | YES |  |  |
| - Bidders shall provide the Service, and Parts Manuals as annotated below. 1 Parts Manual (Per Unit) 1 Service Manual (Per Unit) |  | YES |  |  |